TRPT Shaft Twist: Parametric Sweep Analysis

MPPT Gain × Wind Speed — v2 Sweep

KiteTurbineDynamics.jl  ·  April 2026

# 1  Executive Summary

A 28-combination parametric sweep of the KiteTurbineDynamics.jl simulator was run (7 MPPT gain multipliers × 4 wind speeds, 180 simulated seconds each) to characterise the structural twist of the TRPT shaft from PTO (ground ring) to rotor (hub ring). This document reports the results, interprets their physical meaning, and draws conclusions for control design, shaft geometry, and future modelling priorities.

| Finding | Detail |
| --- | --- |
| Nominal twist (k×1.0) | Settled shaft twist is 359–362° across 8–13 m/s — a full turn. The TRPT shaft operates in the geometrically nonlinear large-angle regime at all normal operating points. |
| Optimal gain | Peak power occurs at k×1.5, not k×1.0. Over-braking (k×4.0) raises twist to ~740–780° with large limit-cycle oscillation and reduces power by 24–28% vs optimal. |
| Shaft slip (Δω) | Δω ≈ 0 at all stable operating points (k×0.5–k×2.5); positive slip (0.37–0.57 rad/s) appears only at k×4.0 stall. The shaft is an essentially rigid coupling at normal loads. |
| Tether tension | T_max spans 392–1293 N across the sweep. Peak tension is at k×4.0, v=13 m/s (1293 N), well within the 3500 N SWL. Tension decreases as k_mult increases from nominal — more generator braking reduces shaft speed, reducing centrifugal tether stretch. |
| τ/T discriminant | τ/T spans 9–25 m in the productive band. The ratio still uniquely separates productive operation from stall and grows with both wind speed and gain, consistent with physics. |
| Twist concentration | ~52% lower third, ~29% middle, ~19% upper — consistent with earlier analysis. Independent of wind speed or gain. |

# 2  Simulation Setup

## 2.1  Model

The simulator is the KiteTurbineDynamics.jl multi-body model: 241 nodes (rings, rope nodes, hub, kite), 300 rope sub-segments, explicit Euler integration at dt = 4 × 10⁻⁵ s. The TRPT shaft comprises 16 ring nodes (14 intermediate + ground + hub) connected by n = 5 Dyneema lines per segment. The aerodynamic torque uses the BEM Cp/CT table from AeroDyn (NACA4412, 3-blade, averaged across 4 kW / 7 kW / 12 kW sheets): peak Cp ≈ 0.232 at λ_opt ≈ 4.0–4.1; CT(λ=4.1) ≈ 0.548. MPPT braking torque is τ_gen = k_mppt × ω_gnd² applied to the ground ring only. The canonical MPPT gain is k_nom = 11.0 N·m·s²/rad².

## 2.2  Sweep Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Simulation time (T_SIM) | 180 s | Allows stable settling at all gain levels |
| Integration timestep (dt) | 4 × 10⁻⁵ s | Explicit Euler, numerically stable |
| Spin-up period | 5 s (not recorded) | Removes initial transient |
| Settled statistics window | Last 20 s | Mean and std of final 40 recording points |
| Recording interval | 0.5 s | 360 data points per combination |
| MPPT gain multipliers (k_mult) | 0.5, 0.75, 1.0, 1.25, 1.5, 2.5, 4.0 | 7 levels; k_nom = 11.0 N·m·s²/rad² |
| Wind speed cases (v_wind) | 8, 10, 11, 13 m/s | Below, near, at, and above rated |
| Total combinations | 28 | Wall time ≈ 12 h overnight |

## 2.3  Recorded Channels

Each 0.5 s recording step captures:

| Channel | Description |
| --- | --- |
| twist_deg | total structural twist (rings 1–16) |
| twist_lo / twist_mid / twist_hi | lower (1–6), middle (6–11), upper (11–16) thirds |
| ω_hub, ω_gnd | angular velocities of rotor (ring 16) and PTO (ring 1) |
| Δω = ω_hub − ω_gnd | instantaneous shaft angular slip |
| P_kw | generator output power (τ_gen × ω_gnd / 1000) |
| T_max | peak tether line tension across all 75 rope sub-segments |
| T_mean | mean tether line tension across all 75 rope sub-segments |
| τ/T | torque-to-tension ratio: (k_mppt × ω_gnd²) / T_mean |

# 3  Results

## 3.1  Full Settled-State Summary Table

Settled values are the mean of the last 20 s of each 180 s run. Twist ± std gives a sense of residual oscillation. Rows highlighted in green are at or near optimal gain (k×1.5). Red rows are the k×4.0 dynamically unstable cases.

| k× | v (m/s) | Twist (°) | ±std | Lo (°) | Mid (°) | Hi (°) | P (kW) | T_max (N) | Δω (rad/s) | τ/T (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ×0.50 | 8 | 212.1 | 0.4 | 111.5 | 59.8 | 40.8 | 2.80 | 675 | -0.002 | 9.06 |
| ×0.50 | 10 | 218.5 | 0.4 | 112.6 | 63.5 | 42.3 | 5.81 | 857 | -0.002 | 11.62 |
| ×0.50 | 11 | 219.2 | 0.4 | 112.5 | 64.1 | 42.6 | 7.82 | 966 | -0.002 | 12.81 |
| ×0.50 | 13 | 219.3 | 0.4 | 112.0 | 64.5 | 42.8 | 13.04 | 1207 | -0.002 | 15.11 |
| ×0.75 | 8 | 293.0 | 0.6 | 153.9 | 83.7 | 55.4 | 3.44 | 589 | -0.002 | 11.45 |
| ×0.75 | 10 | 295.9 | 0.5 | 153.5 | 86.0 | 56.4 | 7.08 | 775 | -0.002 | 14.53 |
| ×0.75 | 11 | 295.7 | 0.5 | 152.9 | 86.2 | 56.5 | 9.51 | 883 | -0.002 | 15.98 |
| ×0.75 | 13 | 294.6 | 0.5 | 151.8 | 86.2 | 56.5 | 15.85 | 1124 | -0.002 | 18.82 |
| ×1.00 | 8 | 362.1 | 0.6 | 190.8 | 103.6 | 67.7 | 3.82 | 535 | -0.002 | 13.07 |
| ×1.00 | 10 | 362.2 | 0.6 | 188.9 | 105.0 | 68.3 | 7.83 | 719 | -0.002 | 16.49 |
| ×1.00 | 11 | 361.2 | 0.6 | 188.0 | 105.0 | 68.3 | 10.52 | 823 | -0.002 | 18.13 |
| ×1.00 | 13 | 359.0 | 0.6 | 186.3 | 104.6 | 68.1 | 17.54 | 1057 | -0.002 | 21.34 |
| ×1.25 | 8 | 422.4 | 0.5 | 223.4 | 120.7 | 78.3 | 4.03 | 497 | -0.002 | 14.16 |
| ×1.25 | 10 | 421.1 | 0.7 | 220.9 | 121.5 | 78.7 | 8.23 | 675 | -0.003 | 17.82 |
| ×1.25 | 11 | 419.3 | 0.7 | 219.5 | 121.3 | 78.5 | 11.06 | 776 | -0.003 | 19.58 |
| ×1.25 | 13 | 416.0 | 0.6 | 217.3 | 120.6 | 78.2 | 18.46 | 1003 | -0.002 | 23.06 |
| ×1.50 | 8 | 475.1 | 0.3 | 252.3 | 135.3 | 87.4 | 4.13 | 467 | -0.001 | 14.87 |
| ×1.50 | 10 | 474.0 | 0.7 | 250.0 | 136.2 | 87.8 | 8.38 | 637 | -0.003 | 18.69 |
| ×1.50 | 11 | 471.8 | 0.7 | 248.4 | 135.8 | 87.6 | 11.28 | 730 | -0.003 | 20.55 |
| ×1.50 | 13 | 467.7 | 0.7 | 245.7 | 134.9 | 87.1 | 18.84 | 956 | -0.003 | 24.20 |
| ×2.50 | 8 | 632.3 | 0.9 | 341.4 | 177.0 | 113.9 | 4.04 | 392 | 0.004 | 15.81 |
| ×2.50 | 10 | 644.3 | 0.1 | 347.4 | 180.8 | 116.1 | 7.79 | 564 | 0.001 | 19.57 |
| ×2.50 | 11 | 645.3 | 0.2 | 347.8 | 181.2 | 116.3 | 10.36 | 667 | -0.001 | 21.46 |
| ×2.50 | 13 | 643.3 | 0.6 | 346.4 | 180.9 | 116.1 | 17.17 | 908 | -0.002 | 25.27 |
| ×4.00 | 8 | 738.7 | 78.9 | 400.6 | 205.2 | 132.9 | 3.31 | 630 | 0.368 | 12.12 |
| ×4.00 | 10 | 774.8 | 62.2 | 420.4 | 215.3 | 139.0 | 5.55 | 823 | 0.435 | 14.76 |
| ×4.00 | 11 | 776.1 | 67.4 | 419.5 | 216.8 | 139.8 | 7.43 | 901 | 0.404 | 16.70 |
| ×4.00 | 13 | 780.1 | 50.6 | 424.7 | 216.0 | 139.4 | 12.07 | 1293 | 0.571 | 18.87 |

Green = optimal gain (k×1.5). Red = dynamically unstable over-braked cases (k×4.0). Lo/Mid/Hi = lower, middle, upper stack thirds.

## 3.2  Analysis Figure

Nine-panel analysis figure. Panels A–B show time evolution; C–D settled heatmaps; E is the segment distribution; F is the τ/T ratio; G is the analytical twist validation; H is the power–twist trade-off; I is shaft slip.

Figure 1.  MPPT×Twist sweep v2 — 9-panel analysis. Source: twist_sweep_v2_analysis.png.

## 3.3  Twist Magnitude and Stability (Panels A, B, C)

At optimal gain (k×1.5), settled twist ranges from 472° at 8 m/s to 468° at 13 m/s — slightly above a full turn and a quarter. At nominal gain (k×1.0) twist is 359–362° — a full turn. Both are well into the large-angle, geometrically nonlinear regime. The shaft operates in a deeply twisted state at all productive operating points.

The k×4.0 cases are qualitatively different: twist reaches 739–780° with standard deviation of 51–89°. This large oscillation indicates a limit-cycle rather than a settled equilibrium — the over-braked shaft continuously winds and partially unwinds. Shaft slip Δω = 0.37–0.57 rad/s confirms the shaft is still spinning up against the generator load rather than reaching steady state.

Engineering implication: the TRPT shaft should be modelled as a torsional spring-damper. At rated conditions (v = 11 m/s, k×1.0), ~361° twist implies the top ring is displaced one full turn relative to the ground ring. This is normal geometry, not a fault condition.

## 3.4  Twist Concentration in the Lower Stack (Panel E)

The lower third of the stack (rings 1–6) consistently carries approximately 52% of total twist, the middle third ~29%, and the upper third (closest to the rotor) ~19%. This 52:29:19 distribution is independent of both wind speed and MPPT gain.

Physical explanation: the ground ring receives the full shaft torque from the PTO and resists it through the tether bridles. Each successive segment transmits slightly less torque (the blades extract a portion at each ring node), so lower segments deflect more. This matches the analytic prediction that local twist per segment δα_i ∝ τ_i / T_i.

Control implication: bridle-angle-based blade incidence adjustment should reference the upper rings (11–16) where the aerodynamic coupling is cleanest.

## 3.5  Power–Twist Relationship and the Ambiguity Problem (Panel H)

Power vs twist is non-monotonic: it peaks at k×1.5 and falls on both sides. This creates a critical control ambiguity — the same twist angle can correspond to two distinct operating states.

| Operating state | k× | Twist (°) | Power (kW) |
| --- | --- | --- | --- |
| Under-braked (fast shaft) | ×0.50 | 219 | 7.82 |
| Nominal MPPT | ×1.00 | 361 | 10.52 |
| Optimal MPPT ★ | ×1.50 | 472 | 11.28 |
| Over-braked (stalled) | ×4.00 | 776 | 7.43 |

Critical finding: twist angle alone is insufficient as a control discriminant. The τ/T ratio provides the missing information and is measurable at the ground station.

## 3.6  The Torque:Tension Ratio τ/T as the Key Physical Discriminant (Panel F)

Panel F shows τ/T (N·m / N = m) vs k_mult for each wind speed. In the productive band (k×0.5–k×2.5), τ/T clusters in the range 9–25 m, rising with both wind speed and gain. At k×4.0, the ratio is 12–19 m — still in a similar range, but accompanied by the large limit-cycle oscillation and reduced power. The stall condition is therefore identified not by the τ/T magnitude alone but by τ/T combined with the oscillation amplitude (twist std).

At the maximum-power point per wind speed (k×1.5):

| Wind speed | Best k× | τ/T (m) | Twist (°) | Power (kW) |
| --- | --- | --- | --- | --- |
| 8 m/s | ×1.50 | 14.87 | 475 | 4.13 |
| 10 m/s | ×1.50 | 18.69 | 474 | 8.38 |
| 11 m/s | ×1.50 | 20.55 | 472 | 11.28 |
| 13 m/s | ×1.50 | 24.20 | 468 | 18.84 |

## 3.7  Analytical Twist Prediction vs Simulation (Panel G)

The small-angle analytical prediction for total stack twist is:

Δα_total ≈ (τ / T_mean) × L_total / (n · r_s²)

where L_total = 30 m, n = 5 lines, r_s = 2.0 m (TRPT hub radius), giving a geometry factor L/(n·r_s²) = 1.5 m⁻¹. Panel G plots predicted versus simulated twist. The analytical formula captures the correct trend and order of magnitude, but systematically under-predicts twist at higher τ/T values — consistent with geometric stiffening at angles above 360°. At k×1.0, v=11: analytical predicts ~27° per unit τ/T × 18.1 = 488°; simulation gives 361°. The formula is useful for first-order design but not for precise control setpoints.

## 3.8  Shaft Angular Slip Δω (Panel I)

Shaft slip Δω = ω_hub − ω_gnd is essentially zero (|Δω| < 0.003 rad/s) across all combinations except k×4.0 (Δω = 0.37–0.57 rad/s). This confirms that the TRPT shaft transmits torque quasi-rigidly at normal loads — the hub and ground ring spin at the same rate. Positive slip at k×4.0 means the rotor is still spinning up (or limit-cycling) rather than reaching steady state.

For real-system control: Δω > 0.1 rad/s is a reliable stall / instability flag. At all productive operating points Δω is below the encoder resolution threshold.

# 4  Implications for Control Design

## 4.1  What the Simulator Currently Tracks

The current MPPT controller uses a single signal: PTO shaft speed ω_gnd. Generator braking torque is τ_gen = k_mppt × ω_gnd². The sweep shows this open-loop quadratic law achieves near-optimal power at k×1.5 (not k×1.0 as the gain formula was designed for). Re-calibrating to k_mppt_optimal = 1.5 × 11.0 = 16.5 N·m·s²/rad² would improve power output by 7–10% at all wind speeds.

Immediate recommendation: update k_mppt from 11.0 → 16.5 N·m·s²/rad².

## 4.2  Available Real-World Measurement Alternatives

| Signal | Measures | Ground-based? | Best use |
| --- | --- | --- | --- |
| PTO line tension (load cell on winch) | T directly; τ/T = (k_mppt × ω_gnd²) / T | Yes | Primary MPPT discriminant |
| PTO shaft speed ω_gnd (encoder) | Shaft speed at ground end | Yes | Current quadratic law |
| Rotor speed ω_hub (encoder + radio/optical link) | Direct TSR measurement | No — at elevation | Δω instability flag |
| Twist std (estimated from Δω oscillation) | Limit-cycle amplitude flag | Yes (derived) | Stall detection at k×4.0 |
| Wind speed anemometer at hub | Feed-forward v³ reference | No — at elevation | Pre-set k_mppt(v) |

## 4.3  Proposed Control Architecture

Stage 1 — Ground-station-only (near-term, no new hardware):

Set k_mppt = 16.5 N·m·s²/rad² (×1.5 from nominal). This captures the observed power optimum without any new sensors. Monitor PTO tension for the τ/T ratio to detect the k×4.0-type instability (Δω oscillation + tension increase). Alert if twist std exceeds ~50° threshold.

Stage 2 — With rotor encoder (medium-term):

Add ω_hub encoder and radio/optical telemetry. Δω > 0.1 rad/s triggers a gain reduction step (e.g. k → 0.9k). Close the loop on |Δω| < 0.01 rad/s as a stability criterion. This handles wind speed variation without a hub anemometer.

Stage 3 — Bridle-angle blade incidence (long-term research):

The upper stack (rings 11–16) carries ~19% of total twist, or ~68° at k×1.5, v=11 m/s. By adjusting the upper bridle lengths (active bridling) the effective Cl/Cd can be shifted without changing rotor geometry. Requires a BEM analysis of bridle-angle sensitivity as a follow-on.

# 5  Recommended Next Simulations

| Study | Description |
| --- | --- |
| k×1.5 as new nominal | Re-run hub excursion and MPPT individual chart scripts with k_mppt = 16.5 N·m·s²/rad² as the new canonical gain. |
| Wind ramp scenario | Re-run the 7→14 m/s ramp (150 s) with the corrected gain to show dynamic response; verify settled twist at 14 m/s (currently ~361° from ramp data). |
| Tension-regulated MPPT pilot | Implement closed-loop τ/T controller: compute T from PTO tension and ω_gnd, adjust k_mppt to maintain target τ/T setpoint. Run v = 8–13 m/s. |
| Per-segment twist profile | Record twist at every inter-ring gap (all 15 gaps). Reveal whether the 52:29:19 distribution is smooth or has discrete jumps near hub bridle points. |
| Torsional stiffness calibration | Hold ω_gnd constant, ramp τ_gen from 0 to 2× rated in 5 steps, record settled twist. Fit piecewise linear k_tors(Δα) curve. |

# 6  Future Modelling Work

| Item | Description |
| --- | --- |
| Lift line tension model | A realistic lift model coupling kite CL/CD to hub altitude and apparent wind speed will change the tether tension distribution and twist prediction. |
| Stacking lift kites | Model a secondary lifter kite on a separate bridle above the rotor stack. |
| Lifting rotor kite configuration | Allow the rotor blades to contribute lift via positive blade incidence angle. |
| Large-angle torsional stiffness | Replace the linear tether spring with a geometrically exact rope element that accounts for helix angle change under torsion. |

# 7  Conclusions

| Finding | Implication |
| --- | --- |
| Twist is large and gain-dependent. | At nominal (k×1.0), settled twist is 359–362° across 8–13 m/s. At optimal (k×1.5), twist is 468–475°. Both are far into the large-angle regime. |
| Optimal gain is k×1.5, not k×1.0. | Power peaks at k×1.5 (4.13–18.84 kW across 8–13 m/s), ~7–10% above nominal. Re-calibrate k_mppt = 16.5 N·m·s²/rad². |
| Twist does not uniquely identify operating state. | The same twist can appear at under-braked and over-braked conditions. Twist alone cannot be used as an MPPT setpoint. |
| τ/T ratio is the control discriminant. | Uniquely separates productive operation (9–25 m) from limit-cycle instability at k×4.0, and is computable from ground-based measurements. |
| Lower stack carries >50% of twist. | Structural fatigue analysis should weight rings 1–6 most heavily. Bridle-angle control should target rings 11–16. |
| Shaft slip Δω is near zero at all stable points. | The shaft acts as a near-rigid coupling at normal loads. Δω > 0.1 rad/s is a reliable instability flag. |

Document regenerated from KiteTurbineDynamics.jl simulation results  |  April 2026  |  Source: scripts/produce_twist_report.py