# TRPT Design Cartography Report

168-hour autonomous design-space campaign
Generated 2026-04-21T09:29:21
KiteTurbineDynamics.jl — Windswept Energy

# Executive summary

The Phase C autonomous campaign ran 60 Differential Evolution islands spanning 2 configurations × 3 beam profiles × 5 axial-curve families × 2 seeds. Each island maintained a feasible-design elite archive of up to 200 unique candidates, giving 12,000 catalogued feasible designs.

Best feasible design across the entire campaign:
  • island     : 10kw_elliptical_straight_taper_s1
  • total mass : 2.808 kg
  • min FOS    : 1.800
  • n_rings    : 5
  • n_lines    : 8
  • r_hub      : 1.600 m
  • axial_idx  : 5
  • knuckle_g  : 10.0

# Phase A — Search-space expansion (complete)

THESIS: The prior 7-DoF linear-taper search box was too small. Real TRPT designs in the Grasshopper archive used elliptic, parabolic, and "straight bottom then grow" axial profiles, so the optimizer needs those curves in its vocabulary. We also promote n_lines (which is identical to n_polygon_sides and n_blades in the topology) and knuckle_mass_kg to free decision variables.

IMPLEMENTATION: src/trpt_axial_profiles.jl adds a five-member axial family — LINEAR, ELLIPTIC, PARABOLIC, TRUMPET, STRAIGHT_TAPER — and a 12-DoF TRPTDesignV2 record. search_bounds_v2() defines the new envelope; objective_v2() is the optimizer-facing scalar cost. ring_radii(design) computes r(z) over uniform z spacing using r_of_z().

FINDINGS: Test suite extends from 308 → 331 assertions (all pass). n_lines = 5, 6, and 7 designs behave very differently — the polygon compression factor 1/(2·tan(π/n)) falls from 0.68 at n=3 to 0.19 at n=8, and the side length 2·r·sin(π/n) halves from n=3 to n=6 at fixed r. Both matter because Euler's P_crit scales as 1/L².

Figure A-1. Polygon family at fixed r_hub — ring shape, side length, vertex compression factor, and P_crit trade under equal mass budget.

# Phase B — DLF calibration and centripetal physics

THESIS: The original OPT_DESIGN_LOAD_FACTOR=0.5 was engineering guesswork. Before we run a serious optimization we must calibrate it against the live multi-body ODE under realistic fault and gust transients, then decide — as an engineering judgement — which envelope designs should actually size against.

EXPERIMENT: scripts/calibrate_dlf.jl ran six scenarios: steady wind at 11 / 15 / 20 / 25 m/s, a coherent IEC-61400-1 gust from 11→25 m/s, and an emergency-brake transient (3× MPPT gain step).

Figure B-1. Per-ring DLF envelope across scenarios (calibrate_dlf.jl).

Figure B-2. Mean/p95/peak DLF per scenario.

FINDING: Peak DLF per scenario — steady25: 0.32, gust 11→25: 0.74, emergency brake: 1.39. The ebrake case dominates the envelope by a factor of 2×.

OPERATIONAL DECISION (Rod, 2026-04-20): hard brakes are avoided by procedure — the rotor is first yaw-stalled via the back-anchor tether before any mechanical braking. We therefore exclude the ebrake case from the sizing envelope and set OPT_DESIGN_LOAD_FACTOR = 1.2 (60% margin over the steady-state worst + 60% margin over coherent gust). This changes from 1.5 → 1.2, saving beam mass.

CENTRIPETAL PHYSICS ADDENDUM: Each vertex now subtracts m·Ω²·r from the inward line load before computing polygon compression. At rated Ω = 20 rad/s and r = 5 m, the blade mass lumped at the hub ring (~2.2 kg / vertex) contributes ~440 N of outward force — not negligible. Knuckle + beam mass at interior rings contributes tens of newtons, small but included for completeness.

# Phase C — Grand parameter sweep

THESIS: Now that the physics is calibrated and the search space is enriched, explore it thoroughly. 60 DE islands (2 configs × 3 beam profiles × 5 axial curves × 2 seeds) each run up to 3 hours and each maintain a 200-element elite archive of unique feasible designs — so the output is a *design space cartography*, not just one winner.

TOP 10 FEASIBLE DESIGNS ACROSS ALL ISLANDS:

| island | mass_kg | FOS | n_rings | n_lines | r_hub_m | axial_idx | knuckle_g |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |
| 10kw_elliptical_straight_taper_s1 | 2.808 | 1.80 | 5 | 8 | 1.60 | 5 | 10.0 |

# Phase D — 2-D/3-D cartography + sensitivity

THESIS: The LHS run feeds 480,000 stratified samples through the physics model and lets us see how feasibility and best mass vary across (n_rings, n_lines, r_hub, axial profile, knuckle) pairs. This is the most honest view of the design space — no optimizer bias.

Figure D-1. 10 kW — min feasible mass heatmap over (n_rings, n_lines). Sweet spot emerges at n_rings ≈ 5-8 with n_lines ≈ 5-7.

Figure D-2. 50 kW — same heatmap. The sweet spot shifts slightly higher in n_rings.

Figure D-3. Feasibility fraction across axial profile × n_lines. Straight_taper and parabolic have highest feasibility.

Figure D-4. 10 kW feasible cloud in (n_rings, n_lines, r_hub) with mass colour.

Figure D-5. 10 kW feasible mass distribution per axial family. Straight_taper and parabolic dominate.

Figure D-6. Spearman rank correlation as a first-order sensitivity proxy — Do_top and r_hub are the dominant mass drivers.

Figure D-7. Mass vs FOS Pareto cloud — the FOS=1.8 floor is a bright wall in the feasibility manifold.

# Phase F — n_lines × knuckle sensitivity

THESIS: n_lines is a topology choice (pentagon vs hexagon vs …) — it changes simultaneously the polygon count, the tether-line count, and the blade count. The LHS data lets us isolate its mass effect.

Figure F-1. Min feasible mass vs n_lines, faceted by config, coloured by beam. Pentagon (n=5) and heptagon (n=7) are best.

Figure F-2. Min mass envelope vs knuckle mass — lighter knuckles always help, but only marginally (< 1 kg swing over the 10-200 g range).

Figure F-3. 10 kW circular — min mass surface in (n_lines, knuckle_mass).

# Phase G — Rich visualisation

# Phase E — Dynamic ODE verification (pending)

Planned: take the top 30 candidates across all islands, rebuild each SystemParams, run the full multi-body ODE for 30 s of steady 11 m/s and 5 s of coherent 11→25 m/s gust, and check that the per-ring FOS remains ≥ 1.5 throughout. Reports which candidates survive dynamic verification — the optimizer uses a quasi-static envelope which may miss transient failures.

# Methodology notes & reproducibility

All data live under scripts/results/trpt_opt_v2/. Each island subdirectory contains log.csv (per-heartbeat), checkpoint.jls (resumable), elite_archive.csv (up to 200 feasible designs), and best_design.json. The LHS cartography lives under scripts/results/trpt_opt_v2/lhs/. Re-running scripts/launch_autonomous_campaign.sh from a clean state reproduces the full campaign.

# Phase I — Torsional Collapse Constraint (v3)

Background: what was missing in v2

All v2 optimisations (Phases C–H) minimised total beam mass subject to a bending/buckling factor of safety FOS ≥ 1.8 at every ring.  Torsional collapse was not constrained.  Torsional collapse occurs when the shaft torque exceeds the critical torque that causes a cylindrical (or conical) shell to buckle under combined axial compression and torsion.  Without this constraint the optimiser exploited highly conical (low taper_ratio ≈ 0.25) geometries, which are dramatically lighter under bending alone but fail torsionally at a fraction of design load.  54 of the 60 v2 winning designs had taper_ratio < 0.9 and would collapse torsionally in service.

Tulloch torsional-collapse criterion

v3 enforces the Tulloch criterion: the torsional factor of safety τ_FOS = τ_critical / τ_applied ≥ 1.5 at every ring.  τ_critical is the Tulloch buckling torque for a thin-walled tube under combined bending and torsion; τ_applied is the TRPT shaft torque at that ring cross-section.  The constraint is evaluated inside the fitness function alongside the existing FOS ≥ 1.8 bending check; any design violating either constraint is marked infeasible and discarded by the optimiser.

What changed: taper forced to cylindrical

The torsional constraint is catastrophic for conical shafts.  A conical TRPT shaft tapers from a large ground-ring radius to a small hub radius; its thin top rings see the same shaft torque as the bottom rings but have far smaller cross-section, making them the governing failure mode.  The only way the optimiser can satisfy τ_FOS ≥ 1.5 across all rings is to make the shaft cylindrical (taper_ratio = 1.0) — confirmed by all 60 v3 designs converging to taper_ratio = 1.000.

Quantitative mass impact

Table I-1 summarises v2 vs v3 masses for all 15 beam×axial combinations at 10 kW and 50 kW.  The 10 kW circular straight-taper winner increased from 2.81 kg (v2, taper=0.25) to 15.44 kg (v3, taper=1.0) — a +449 % increase. The mean mass increase across all 60 configurations is +304 %.  Six 50 kW airfoil configurations already had taper_ratio = 1.0 in v2 and are unaffected (0 % change).  All others are significantly heavier under the physical constraint.

| Config | Beam | Axial | v2 mass (kg) | v3 mass (kg) | Δ mass % | v2 taper | v3 taper | v3 Tor FOS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table I-1.  v2 vs v3 mass and geometry comparison for all 60 configurations (values averaged over two random seeds; taper column shows mean).

The taper_ratio = 1.0 finding and its physical meaning

A cylindrical TRPT shaft has constant cross-sectional moment capacity from ground to hub.  This means the shaft wall, once sized to resist the peak torque at the ground ring, is over-designed at every higher ring — the shaft carries excess material throughout its length.  This is a fundamental mass penalty imposed by the torsional physics.  Future optimisation must explore alternative structural strategies (variable wall thickness at constant outer diameter, composite layup, lattice rings) to recover some of the mass advantage of v2 without violating torsional collapse.

Practical implication for ground ring deployment

A cylindrical shaft means that all rings have the same radius.  This significantly simplifies ground ring deployment: the ground ring (the lowest ring, previously the widest in a conical design) is now the same diameter as all upper rings.  Ground logistics (transportation, anchoring radius, swept-area footprint) are unchanged relative to the hub radius rather than expanding to a wide base.  The torsional constraint thus has a practical benefit for site deployment, even though it increases mass.

Figures — Phase I

[Image not found: fig_v2_vs_v3_mass_comparison.png]

Figure I-1.  Mass impact of enforcing the Tulloch torsional collapse constraint.  Grey bars: v2 (no torsional constraint).  Teal bars: v3 (τ_FOS ≥ 1.5).  Percentage labels show mean mass increase per combination. The 50 kW airfoil elliptic/linear/trumpet bars are equal height because those v2 designs already had taper=1.0.

[Image not found: fig_v3_geometry_shift.png]

Figure I-2.  Geometry change driven by the torsional constraint.  Left: taper_ratio scatter — all 60 v3 designs sit at taper=1.0 (red dotted line) regardless of v2 taper, confirming the constraint forces cylindrical geometry.  Right: hub radius shift — v3 r_hub is consistently larger than v2 r_hub as the optimiser compensates for the cylindrical constraint by growing the shaft radius.

[Image not found: fig_v3_winner_glmakie.png]

Figure I-3.  GLMakie render of the v3 10 kW winner: circular straight-taper, 15.44 kg, FOS=1.8, torsional FOS=1.5, 5 rings × 8 lines, r_hub=1.99 m, taper=1.0 (cylindrical).  Beam outer diameter colour-coded (viridis); knuckle vertices as red spheres; tethers as dark grey lines.